Programmer test log – AQUATOXVolumeModel, JSON vs. AQUATOX 3.2

JSON test logs in the hms_backend\Stream.Hydrology\AQUATOX\TEST folder

12/20/2017, AQUATOXVolume model reproduces results from East Fork Poplar Creek TN from AQUATOX 3.2

Results from AQUATOX_Volume_Model_PTest1.JSON above

Results from AQUATOX 3.2

1/26/2018, testing of AQUATOX trapezoidal integration and instantaneous output

100-day timestep

Results from AQUATOX_Volume_Model_PTest2.JSON below

Results are essentially identical  -- AQUATOX does print final time step though it does not have a 100-day averaging period.

1/26/2018, test instantaneous conc vs. AQUATOX output

Results from AQUATOX_Volume_Model_PTest3.JSON above (re-run 3/9/2018)

1/26/2018, test hourly output, run for two days – results evenly spaced and consistent with trapezoidal integration,  (30 day test in AQUATOX_Volume_Model_PTest4.JSON)

And with instantaneous concs (1 year test in AQUATOX_Volume_Model_PTest5.JSON)

3/9/2018, Re run of AQUATOXVolume model using new JSON format (JSON.NET):  Model reproduces results from East Fork Poplar Creek TN from AQUATOX 3.2

Results from AQUATOX_Volume_Model_PTest1.JSON above – re-run 3/9/18

Results from AQUATOX 3.2

1/26/2018, testing of AQUATOX trapezoidal integration and instantaneous output

100-day timestep

Results from AQUATOX_Volume_Model_PTest2.JSON below – re-run 3/9/18

Results are essentially identical  -- AQUATOX 3.2 does print final time step though it does not have a 100-day averaging period.  HMS does not.

1/26/2018, test hourly output, run for thirty days – results evenly spaced and consistent with trapezoidal integration,  (30 day test in AQUATOX_Volume_Model_PTest4.JSON) – re-run 3/9/18

And hourly test with instantaneous concs (1 year test in AQUATOX_Volume_Model_PTest5.JSON) – re-run 3/9/18

Lake Jesup FL Simplified_NoResults.aps, Lake Jesup FL trimmed2.JSON

Ammonia, Nitrate, TSP Precisely matched 4/2/2018

Lake Jesup FL Simplified_NoResults.aps, Lake Jesup FL trimmed2.JSON

Oxygen, CO2 Precisely matched 4/2/2018

Lake Jesup FL Simplified_NoResults.aps, Lake Jesup FL trimmed2.JSON

Water Volume Precisely matched 4/2/2018

Added linkages to AQUATOX animal / plant linkages:

- NH3 & NH4+ ASSIM_JSONLINK (mg/L d)

- NH3 & NH4+ ANREMIN_JSONLINK (mg/L d)

- NH3 & NH4+ PLREMIN_JSONLINK (mg/L d)

- NH3 & NH4+ OMREMIN_JSONLINK (mg/L d)

- NO3 ASSIM_JSONLINK (mg/L d)

- TOT. SOL. P ASSIM_JSONLINK (mg/L d)

- TOT. SOL. P ANREMIN_JSONLINK (mg/L d)

- TOT. SOL. P PLREMIN_JSONLINK (mg/L d)

- TOT. SOL. P OMREMIN_JSONLINK (mg/L d)

- CO2 DETDECMP_JSONLINK (mg/L d)

- CO2 RESPIRATION_JSONLINK (mg/L d)

- CO2 CO2ASSIM_JSONLINK (mg/L d)

- OXYGEN PHOTOSYN_JSONLINK (mg/L d)

- OXYGEN RESPIRATION_JSONLINK (mg/L d)

- OXYGEN NITRIFIC_JSONLINK (mg/L d)

- OXYGEN CBOD_JSONLINK (mg/L d)

Inputs are integrated over a day’s AQUATOX solution (stair-stepped) whereas when all state variables are included, inputs are included in sub-day timestep differential equations (4th & 5th order solutions)

Oxygen and CO2 predictions are pretty much identical in magnitude and pattern

Top is LJ_Complex2.JSON bottom is  Lake Jesup FL.aps

Results are close but not precise, especially TSP and NO3.  I will spend some time to determine if this is a function of step nature of derivative rates or another error.

This graph shows the lack of precision between NO3 and TSP results.  I will run hourly simulations and save rates hourly to see if this increases precision.

Tested hourly time steps in both simulations-- no additional conformance.   Passed hourly rates in – a good test of hourly-rate interface, but NO3 results are not significantly different: LJ_Complex3_hourlyNO3.JSON

Fixed a problem with JSON writing/reading loadings multipliers.  This made results closer but still not precise.  Looking into TSP discrepancy next.

Missing Calcite precipitation linkage.  When added in, makes the results even closer!

4/4/2018: Confirmed that differences are caused by passage of daily data for intermediate derivative rates (e.g. assimilation rates for nutrients from algae).   Switched to 1/10th of a day passage of data between AQUATOX 3.2 and AQUATOX HMS and the differences between the simulations disappear

(shortened simulation period as passage of data becomes burdensome.)

Lake Jesup FL_0.1d_rates.aps compared to LJ_Complex3_hourly_rates.JSON